Technical tip 1

Cable sizes and current properties

| Conductor (mm) | Cross section (mm2) | Current rating (amps) | Cable diameter (mm) | Max load 12 volt watts | Max load 24volt watts | Resistance per metre at 20*c (ohms) |
| --- | --- | --- | --- | --- | --- | --- |
| 9/0.30 | 0.65 | 5.75 | 2.5 | 69 | 138 | 0.0294 |
| 14/0.30 | 1.00 | 8.75 | 2.7 | 105 | 210 | 0.0189 |
| 21/0.30 | 1.5 | 12.75 | 3.1 | 153 | 306 | 0.0125 |
| 28/0.30 | 2.0 | 17.5 | 3.4 | 210 | 420 | 0.0094 |
| 35/0.30 | 2.5 | 21.75 | 3.8 | 60 | 520 | 0.0075 |
| 44/.30 | 3.0 | 27.5 | 4.2 | 330 | 660 | 0.0060 |
| 65/0.30 | 4.5 | 35.0 | 5.1 | 420 | 840 | 0.0041 |
| 84/0.30 | 6.0 | 42.0 | 5.9 | 500 | 1000 | 0.0031 |
| 97/0.30 | 7.0 | 50.0 | 6.2 | 600 | 1200 | 0.0027 |
| 120/0.30 | 8.5 | 60.0 | 7. | 720 | 1440 | 0.0022 |
| 80/0.40 | 10.00 | 70.00 | 7.5 | 840 | 1680 | 0.0018 |

Battery/starter cable sizes

| Conductor cross section (mm) | Current rating (amps) |
| --- | --- |
| 16mm | 110 |
| 20mm | 135 |
| 25mm | 170 |
| 35mm | 240 |
| 40mm | 300 |
| 50mm | 345 |
| 60mm | 415 |
| 70mm | 485 |

How to calculate amperage

Divide wattage by voltage equals amperage eg:

3500 watts divided by 12 volts = 291 amps

3500 watts divided by 24 volts = 146 amps

Also allow 50% or current to be safe therefore:

3500 watts divided by 12 volt = 291 amps x 50%  =  436 amps

3500 watts divided by 24 volt  = 146 amps x 50%  =   219 amps